Sample specification from The Home Owner's Guide to Building Without Regret (Appendix D). Provided for reference only. It isn't a contract document, and the inclusions, allowances and prices are examples from past Queensland projects. constructor.net.au/book-resources

| CLIENT: HOUSE TYPE: Residential House Construction |  | COUNCIL: GOLD COAST CITY COUNCIL COVENANT: YES | CONTRACT DATE: START DATE: |
| --- | --- | --- | --- |
| QBCC Insurance policy no:# | BA:# | PA:# | Certifier Ref:# |
| INCLUSIONS |  |  |  |
| Inclusions | Items marked “Not Included” and “struck through” are not included in the house costings. They are noted on here so you can clearly see what is and isn’t included rather than not mention anything at all to you. |  |  |
| Site Services | All costings are based the site being “fully serviced” | No allowance has been made unless otherwise noted to provide services to site - such as power, water, phone, sewer, stormwater, fibre, cable, internet, telephone, gas or otherwise. |  |
| INSURANCES, FEES & WARRANTIES |  |  |  |
| QBCC home warranty insurance | Included |  |  |
| PLSA Levy Fee | Included |  |  |
| Contractors All Risk Insurance | Included |  |  |
| Council Vehicle Cross-Over Fees (GCCC only) | Included | Note: application fees excluding relaxations, town planning |  |
| Council Plumbing Fees | Included |  |  |
| Certification Fees | Included |  |  |
| 6-month maintenance inspection (builder) | Included | Note: this is the builder's 6-month maintenance inspection. The defects liability period under the contract runs 12 months from practical completion (statutory warranties run longer). |  |
| Structural Guarantee | Included |  |  |
| EXCAVATION & PREPARATION WORKS |  |  |  |
| Topsoil Removal & Disposal | Top 50mm of grass over entire site area to be removed and disposed of | Allow for all topsoil to be scraped and removed from site |  |
| Slab prep & filling | Machine Hire and labour to place and grade fill against slab edge around house perimeter | Allow for mini loader to prep soil to slab edge perimeter |  |
| Sediment & erosion control | Sediment fence installation | Includes: sediment fence to front of property, maintained throughout construction, |  |
| All weather driveway access pad | Earthmover to supply and install at Earthworks stage prior to sediment fence installation | All weather driveway (50mm gravel) trenched below top of kerb & channel (1 load only allowed) |  |
| SITE AMENITIES, SAFETY, CLEANING |  |  |  |
| Site WC | Portable | For duration of construction period, serviced fortnightly |  |
| Site rubbish bins | Skip | Allow for 9m3 skips throughout construction or sufficient depending on available site storage space |  |
| Building debris removal | Included | Allow for two bobcat site cleans into skip bins |  |
| Excess soil removal | Included |  |  |
| Builders & safety signage | Included |  |  |
| Builders house cleans (internally & externally) | Included | Allow for main plus three trips (re-cleans) Note: Includes windows inside & out, tracks |  |
| House wash & external path/driveway clean | Included | Allow for full washdown after landscaping |  |
| UNDERSLAB PLUMBING & DRAINAGE |  |  |  |
| Plumbing & Drainage Fees | Included |  |  |
| Sewer | Product: UPVC pipework | Size: 100mm pipework diameter & fittings Quantity: 50lm included as standard |  |
| Stormwater | Product: UPVC (below ground) & PVC (above ground) pipework | Size: 100mm pipework diameter & fittings Quantity: 50lm included as standard |  |
| Sand bedding | Included: due to H Class site | Locations: Under slab sewer & stormwater |  |
| Articulation joints & flexible couplers | Included: due to H Class site | Location: To sewer & stormwater underslab pipework |  |
| Stainless Steel Strapping To Services | Included | Location: Per engineers report Type: Stainless steel strap from service pipe to fabric | Note: Strap to be taut not loosely tied |
| Cold water pipework & connection | Product: Polymer pressure pipework | Location: All internal fittings & outlets connected to mains Size: 19mm pipework Quantity: 10lm included from meter to house location |  |
| FOUNDATIONS |  |  |  |
| Soil Test | Included | Location: Two boreholes, diagonal from each other, located based on proposed house sitting in relation to site boundaries. |  |
| Classification | Prices are based on “H” Class soil type | Note: in accordance with Engineers Report Classification |  |
| Foundation Design | Included |  |  |
| Inspections & Certification | Included |  |  |
| Concrete Pump | Included | When: Piers & footings, slab Quantity: 2x pump set ups |  |
| Foundations & slab | Concrete slab system designed for “P” soil classifications to AS2870 with no brick rebate. Step down to Garage floor to suit driveway gradient. | Slab thickness: 100mm Pier depth Included: slab/footing piers up to 30lm Bore Size: 300mm diameter bored piers included. | Slab Finish: Helicopter (trowelled) Finish Edge Finish: Vibrated to Class 4 or sponged finished |
| GENERALLY |  |  |  |
| Wind Category | Rating: N3 |  |  |
| AHD Requirements | Included | Note: Surveyor Certification finished floor level as constructed in accordance with AHD requirements for site. | Note: Includes surveyor marking earthworks stake/reference point for earthworks & footing contractor set outs. |
| Vegetation Management | Note: No activity to be performed in easements at rear of property |  |  |
| Bushfire Requirements | Not Included |  |  |
| Acoustic Requirements | Not Included |  |  |
| Tree Removal | Not Included |  |  |
| Soil stabilisation & retention | Not Included |  |  |
| Rock removal - excavation | Not included |  |  |
| Site Access | Accessibility to the building site is the responsibility of the Owner. | Note: Any costs incurred by the Builder due to difficult site access including crane hire and hand carting of materials to be paid by the Owner. |  |
| Hand Carting | Not Included | No allowance has been made for hand carting due to difficult site access |  |
| Crane Hire | 20 t pick-and-carry mobile crane | Allow for five site visits (upper frame, trusses, steel) |  |
| Retaining Walls | Not included | Note: Engineered drop slab edge is included as detailed on plans. |  |
| TERMITE PROTECTION |  |  |  |
| Termite protection | Material: Termite treatment to AS3660.1 (Section 6) including monolithic slab and physical barrier system to service preparations and perimeter. | Locations: perimeter and penetrations, piers | Note: supplier warranty provided |
| Treated timber framing | Material: H2 classification treated timber by supplier | Location: trusses & frame timbers | Note: annual inspection required to maintain warranty |
| SAFETY |  |  |  |
| Site safety packet & information - WH&S | Included |  |  |
| Site specific plan - WH&S | Included |  |  |
| Periodic Safety Inspections | Included |  |  |
| Perimeter Scaffolding | Included | Location: Entire perimeter scaffolding to standard height, 1.8m under eaves of upper floor levels |  |
| Hang on platform for fascia & gutter installation - upper roof | Included | Perimeter scaffold performs this function |  |
| Edge protection - roof & cladding installation - lower roof | Included | Perimeter scaffold performs this function |  |
| Walk around platform to lower roof - cladding installation | Not Included |  |  |
| Perimeter guard rail system - second floor platform | Not Included | Perimeter scaffold performs this function |  |
| Temporary Fence Hire | Included | Duration: Entire site fenced until lockup stage reached |  |
| EXTERNAL FINISHES & FIXTURES |  |  |  |
| External Concrete Driveway & Paths, Porch Slab (CONC1) | Manufacturer: Premixed concrete supplier | Type: Exposed aggregate NOTE: Poured with driveway Note: 50mm crusher dust with plastic membrane slip sheet above (reactive site) Mesh: F82 | Colour: Black Night Finish: Acid wash and sealer applied after curing time observed |
| External Ceramic Tiles (PAV1) | Material: Ceramic Tile | Location: Alfresco, Deck, Bed 3 Deck |  |
| Hot water system | Brand: Builder's range | 32 L/min gas continuous-flow (instantaneous) hot water system | Note: includes recessed wall box |
| Clothes line | Brand: Builder's range Material: Stainless steel wire frame | Type: Wall-mounted fold-down Size: 2.2m x 1.5m | Colour: Black |
| Letterbox | Brand: Builder's range (or similar) | Wall mounted through masonry fence near to gatehouse |  |
| Retaining walls | Masonry block build up (400x200 blocks) | Location: Rear elevation of house | Finish: Rendered & Painted Colour: refer to paint selection below |
| Block walls & landscape structures | Masonry block build up (400x200 blocks) | Location: Gatehouse, piers, front streetscape elevation | Finish: Rendered Painted Colour: refer to paint selection below |
| External stairs | Galvanised stringers - aluminium treads | Location: Alfresco to Deck | Finish: Refer to paint selection below |
| Air Conditioning | Brand: Ducted reverse-cycle system, Builder's range or similar Type: Ducted, two units one upper, one lower Size: 3 Phase 13kw | Wall Mounted Linear Style: Rectangular plastic Wall Mounted Linear Locations: Kitchen bulkhead (x2 faces), Media Bulkhead, Bed 2, Bed 3, Bed 4, Master Bed Ceiling Outlet Style: Round Ceiling Outlet Locations: Passage from entry, Butlers Pantry, Dining, Study Nook Area, Rear Passageway | Finish: Manufacturer finished |
| Spa pump & cover | Not Required or Included |  |  |
| Garage door | Material: pre-painted steel slimline door to the front facade of the garage Brand: Builder's range | Profile: TBC Size: 4800mm wide x 2400mm high | Colour: Surfmist |
| Garage door remotes | Manufacturer: To match garage door motor (or similar) Note: Three (3) handsets included | Type: One wall mounted and two hand held remotes Location: Wall mounted in Garage by house access door | Note: Mounted above light switch plate |
| Privacy Screen (PS01) | Material: Aluminium Privacy Screen | Type Fixed louvre privacy screen Location: Deck | Colour: Surfmist |
| TIMBER FRAMING |  |  |  |
| External & Internal Walls | Material: Pine timber framing Strength: M12 throughout | Size: 90mm FTC Height: 2720mm (lower & upper) Treated: H2 classification Type: stick frame | Note: no prenail (lead time is excessive) & frames rubbish |
| Trusses | Pine timber framing | Size: As designed & certified by truss manufacturer Treated: H2 classification Type: Pre-Nail | Note: Certification provided by manufacturer |
| Eaves timber | Pine Timber Framing | Size 70x35mm Treated: H2 Classification | Note: NO 50x38 to be used |
| Stud Centres | Centres: 450mm & 600mm | Spacing: 450mm cns Location: Wet areas, Kitchen, All Robes Spacing: 600mm cns Location: All other areas |  |
| Abutting wall frames fixing method | Note: 3 points required by code | Type: Fix full stud not blocks to external & internal intersections |  |
| Upper level flooring | Type: Structural particleboard flooring or similar as specified by manufacturer | Notes: As per manufacturer design |  |
| NOGGING HEIGHTS (fixture fastening) |  |  |  |
| Ensuite | 180mm timber noggings cut & fixed horizontally between studs | Towel Rail: 1000mm CL above FFL WC Roll: 700mm CL above FFL Robe Hook: 1900mm above FFL Vanity Unit Noggings (wall mounted): 400mm @ 650mm above FFL - TBC |  |
| Bathroom | 180mm timber nogging cut & fixed horizontally between studs | Towel Rail: 1000mm CL above FFL WC Roll: 700mm CL above FL Robe Hook: 1900mm above FFL Vanity Unit Noggings (wall mounted): 400mm @ 650mm above FFL - TBC |  |
| WC | Material: 90mm timber nogging cut & fixed horizontally between studs | Location: 900mm CL above FFL |  |
| WIR | Material: Studs at 450mm centres from corners |  |  |
| All Robes | Material: 90mm timber nogging cut & fixed vertically between studs Studs at 450mm centres from corners | Horizontal nogg @ 1850 CL above floor level |  |
| Kitchen | Studs at 450mm centres from corners |  |  |
| TV Noggings | Material: Nogg bays between two adjacent studs (1200mm wide) | To all bedrooms & media room on assumed/noted TV wall mount point |  |
| EXTERNAL WALLS & FINISHES |  |  |  |
| Brick veneer walls (BRK) | Not Included |  |  |
| Brick veneer wall mortar colour & joint type | Not Included | Generally - Mortar Type: Sand : Cement : Lime Generally - Joints: Ironed Finished | Colour: Not Required |
| Blockwork (BLK) | As per plan to engineers details | Mortar Type: Sand : Cement : Lime Joints: Flush Location: Front elevation fence | Colour: Natural |
| Weephole Void Formers (if required) | When Required: Plastic weephole void formers | Type: Standard |  |
| Cavity Ties (if required) | Masonry wall ties | Type: Stainless Steel |  |
| Expansion Ties (if required) | Masonry wall ties | Type: Stainless Steel |  |
| Render finish (RF1) | Flexible render system | Type: Sponge finish Thickness: 3mm build up | Note: Acrylic based polymer render NO Sand & cement only render Colour: Refer to paint schedule below |
| Fibre cement cladding (FC1) | Fibre cement | Type: Render-base fibre cement sheet | Colour: Refer to paint schedule below |
| Fibre cement cladding (FC2) | Fibre cement | Type: Shiplap-edge fibre cement panel | Colour: Refer to paint schedule below |
| Fibre cement cladding (FC3) | Fibre cement | Type: Fibre cement weatherboard | Colour: Refer to paint schedule below |
| Fibre cement cladding (CT1) | Fibre cement | Profile: Slimline weatherboard | Colour: White |
| Fibre cement cladding (CT2) | Fibre cement | Profile: Slate-look textured panel | Colour: Slate |
| ROOF & RAINWATER GOODS |  |  |  |
| Metal Roof (MR1) | Material: Pre-painted steel roof sheeting, trapezoidal profile | Roof pitch as per plans, eaves as per plan width | Colour: Colorbond Surmist |
| Metal Flashing (FL) | Location: Above Garage/Parapet, Gatehouse, Porch, Rear Elevation |  | Colour: Colorbond Surfmist |
| Skylight (SKY1) | Not Included |  |  |
| Fascia (MF) | Material: Pre-painted steel Note: By Roofer | Type: Smooth finish | Colour: Colorbond Surfmist |
| Roof Ventilators (RV1) | Roof ventilator | Size: 450mm Location: Spaced equally across main roof area length | Colour: Surfmist Note: 3x off roof vents |
| Gutters (G) | Material: Pre-painted steel | Profile: 150mm Quad | Colour: Colorbond Surfmist |
| Leaf Guards | Not Included |  |  |
| Downpipes (DP) | Material: Metal Supplied by: Roofer Installed by: plumber | Size: 100mm Profile: Round with 90mm standoff brackets | Colour: Zinc-aluminium coated finish with matching standoff brackets |
| Downpipe brackets | Material: Metal Type Standoff | Size: 90mm standoff Centres: 1500mm maximum vertical centres to brackets | Colour: Zinc-aluminium coated finish |
| Gutter Guard (GG) | Not Included |  |  |
| Planter Drainage (PL1) - Not Required | Not Included Material: Plastic drainage cell wrapped in geotextile fabric. | Locations: Not required | Note: Allow for separate geotextile fabric sheet to base and sides of planter. |
| EAVES/SOFFIT LININGS |  |  |  |
| Eaves linings (SOF1) | Brand: Builder's range Material: Fibre cement sheet | Size: 4.5mm Trims: PVC joint mould between sections | Finish: Painted Colour: Refer to paint schedule below |
| Eaves Vents | Brand: Builder's range Material: Fibre cement sheet | Size: 4.5mm Trims: PVC joint mould between sections Location: as per plan | Finish: Painted Colour: Refer to paint schedule below |
| WATERPROOFING MEMBRANES, BEDDING & FLOOR WASTES |  |  |  |
| Waterproofing membrane wet areas | Product: Self-adhesive polyester-reinforced sheet membrane, 1.8mm Membrane-compatible steel angles and terminations | Locations: Laundry, Powder, WC, Bath, Ensuite Note: Waterproofer to leave door stops onsite for carpenter installation square to jambs once fixed in position |  |
| Waterproofing membrane deck areas | Product: Self-adhesive polyester-reinforced sheet membrane, 1.8mm Membrane-compatible steel angles and terminations | Location: Deck (Master Suite, Deck (Bed 3) | Note: Areas receiving membranes to be clean and parged with falls to outlets. |
| Wet area screed/bedding | Material: Sand, cement, acrylic polymer (acrylic bonding additive) | Size: 50mm Locations: Bath, Bath Shower, Ensuite, Ensuite Shower, Ensuite WC | Note: keep bath platform level so bath is installed level then fall floor to waste |
| Floor wastes | Product: 100mm tile-insert floor waste Brand: TBC | Location: Powder Floor, Bathroom floor, Ensuite Floor, Dry Waste to: Laundry Floor Only |  |
| Floor strip drains | Product: 100 strip drain channel Brand: Builder's range Material: Stainless steel | Location: Bath Shower, Ensuite Shower Type: Tile over lip style, against wall Size: as available from manufacturer to fit within shower width and to compliment shower floor tile size | Colour: Stainless steel |
| Membrane Protection: under outdoor paving | Heavy duty polyethylene slip sheet. | Location: Not Required |  |
| Membrane Protection: In planter bases | Product: Plastic drainage cell | Location: Not Required | Note: Allow for separate geotextile fabric sheet to base and sides |
| Membrane Protection: In planter sides | Brand: Builder's range Material: Fibre cement sheet | Location: Not Required Size: 6mm fibre cement sheet |  |
| SARKING & INSULATION |  |  |  |
| Roof Insulation: Foil-faced roof blanket | Brand: Builder's range Material: Glasswool or similar with bonded foil blanket | Location: 50mm to entire roof area Type: Bonded Blanket | Note: Glasswool insulation batts to ceiling to meet 6 Star Energy Rating requirements. |
| Metal wall cladding insulation | Brand: Builder's range Material: Glasswool or similar with bonded foil blanket | Location: 50mm to areas behind metal wall cladding Type: Bonded Blanket | Note: Glasswool insulation batts to ceiling to meet 6 Star Energy Rating requirements. |
| Ceiling Insulation | Brand: Builder's range Material: Glasswool or similar | Location: R2.5 to accessible ceiling spaces Type: Bulk Insulation | Note: Glasswool insulation batts to ceiling to meet 6 Star Energy Rating requirements. |
| External Wall Insulation | Brand: Builder's range Material: Glasswool or fire-rated mineral wool batts (Garage external wall) | Location: Internal walls (All except garage zero lot wall) R2.0 Location: Garage external zero lot wall Firerated batts Type: Bulk Insulation | Note: Glasswool insulation batts to ceiling to meet 6 Star Energy Rating requirements. |
| Internal Wall Insulation | Brand: Builder's range Material: Glasswool or similar | Location: Internal walls (All) R2.0 Type: Bulk Insulation | Note: Glasswool insulation batts to ceiling to meet 6 Star Energy Rating requirements. |
| Wall Wrap | Type: Builder's range (or similar) Material: Reflective foil vapour barrier | Location: All external walls Type: Anti-glare reflective foil | Note: Reflective foil wrap to external walls to meet 6 Star requirements. |
| Fire Check | Type: Fire-rated mineral wool batts Material: | Location: Garage zero lot boundary wall Type: Bulk insulation | Note: photograph installed product for certifier records |
| EXTERNAL DOORS, WINDOWS & FITTINGS |  |  |  |
| Aluminium windows | Brand: Builder's range | Type: Residential aluminium window suite | Finish: Clear Anodised Colour: Clear Anodised |
| Aluminium doors | Brand: Builder's range | Type: Residential aluminium door suite | Finish: Clear Anodised Colour: Clear Anodised |
| Aluminium doors & windows glass & tint colour | Brand: Builder's range | Type: 6.38mm laminated | Finish: Tinted Colour: Grey Tinted |
| Wet area glazing | Brand: Builder's range | Type: Low-E laminated glass Style: Translucent satin laminated | Finish: Translucent satin Colour: Obscure & translucent satin glass does not come tinted |
| Front Entry Door & Frame | Brand: Builder's range | Type: French Style Hinged | Finish: Clear Anodised Colour: Clear Anodised |
| Laundry Door & Frame | Brand: Builder's range | Type: French Style Hinged | Finish: Clear Anodised Colour: Clear Anodised Reference: |
| Security Screens | Not Included |  |  |
| Fly Screens | Not Included |  |  |
| Curtains | Not Included |  |  |
| Blinds | Not Included |  |  |
| STEELWORK |  |  |  |
| Generally | Finish: Galvanised | Note: All exposed steelwork to be site paint finish over shop hot dip galvanizing. Code:-AUS 13393 |  |
| Finish | Brand: Protective coating system (to match buildup thickness) | Primer: Surface-tolerant epoxy primer. Intermediate: High-build epoxy intermediate coat Top Coat: High-build acrylic topcoat | Colour: Refer to paint schedule below Application: Refer to specification section 1600 for preparation details. |
| External Nuts & Bolts | Finish: Galvanised | Notes: all external nuts & bolts to be galvanised finish | Note: Cup head bolts to timber fixings, Hex head bolts to metal to metal fixings |
| STRINGERS, HANDRAILS & BALUSTRADES |  |  |  |
| Stair Stringers | Material: DAR Jarrah with Pencil Round Durability: Class 1 | Location: Internal stairs Size: 240x45mm | Finish: Painted Colour: Refer to paint schedule below (clear coated) |
| Handrail (HR1) | Material: Stainless Steel | Location: Internal stairs, Bed 3, Alfresco, Upper Deck Size: 42x19mm Fixing: round stainless steel patch fixings | Finish: Polished Colour: Refer to paint schedule below |
| Timber Handrail (THR1) | Material : Timber - Jarrah Durability: Class 1 | Location: Alfresco Stairs Profile: Breadtop/Ladies Waist | Finish: Clear Matt Finish Colour: Refer to paint schedule below |
| METAL GATES & FENCES |  |  |  |
| Metal Gate (MTG1) | Material: Aluminium square section, 1800mm high Style: Privacy screen type infill | Location: Gatehouse & one each to rear elevation Notes: Heavy Duty Hinges Keyed: Alike to house | Finish: Colorbond Surfmist Keyed lock with auto latch |
| Metal Gate (MTG2) | Material: Aluminium square section, 1800mm high Privacy screen type infill | Location: Driveway Type: Track sliding style gate or hinged (depends on access) | Finish: Colourbond Surfmist Notes: Auto unit motor included Handsets: three (3) |
| Concrete Boundary Fence (CF1) | Manufacturer: PRECAST CONCRETE - 100mm thick | Location: To side boundaries | Finish: Painted Concrete Colour: Lexicon |
| Pre-painted steel gate (CG1) | Manufacturer: Landscaper selected & supplied | Location: Entry side elevation | Finish: Pre-painted steel Colour: Colorbond Dune |
| LANDSCAPING & EXTERNAL CONCRETE |  |  |  |
| *Landscape designer's concept design & landscaper's quotation refers* | Plant schedule | Note: Refer to landscape design & schedule |  |
| LIGHTING & ELECTRICAL SERVICES - GENERALLY |  |  |  |
| Internal Light Sensors - NOT INCLUDED | Electrician supplied | Locations: As per electrical plan Profile: Dome style - 360 degree or similar depending on fixing location and view obstruction | Note: Manual override switch to each room |
| External Light Sensors | Product: Electrician supplied | Locations: As per electrical plan Profile: Dome style - 360 degree or similar depending on fixing location and view obstruction |  |
| Power cable (lead in) | Underground connection | Capacity: sized by electrician with growth capacity Size: Single phase |  |
| Meterbox | Brand: Builder's range or similar | Location: As per plan | Finish: Painted Colour: see paint schedule below |
| Circuit Breakers & ELCB | Brand: Builder's range or similar | Location: Meterbox Capacity: Electrician specified |  |
| Sub Board | Brand: Builder's range or similar | Location: Entry - back to back with meterbox | Finish: Manufacturer Colour: White |
| Data / Comms | Brand: Builder's range | Type: Slimline switch plate range with LED Locations: To all bedrooms, media, dining & kitchen, butlers pantry, study alcove, above WIR shelf (security), deck & alfresco | Finish: Manufacturer Colour: White |
| Phone points | Brand: Builder's range | Type: Slimline switch plate range with LED Locations: To all bedrooms, media, deck, alfresco, living & dining Connection: house internally wired to external connection point with lead into telecommunications pit with draw wire | Finish: Manufacturer Colour: White |
| TV/Pay TV | Brand: Builder's range | Type: Slimline switch plate range with LED Locations: To all bedrooms, media, deck, alfresco, living & dining Connection: Internally wired to external house connection point awaiting client connection | Finish: Manufacturer Colour: White |
| Security System | Product: By security installer Brand: TBC | Type: Wired alarm panel with keypads or similar Location: Ceiling mounted sensors “dome style” Keypad locations: Garage beside cavity door, Entry, Master Bed Quantity: Three (3x) off | Notes: Same as light sensor except when security activated triggers motion detection alarm |
| Smoke Detectors | Brand: Builder's range or similar | Type: TBC Locations: As per plan | Finish: Manufacturer Colour: Manufacturer |
| TV Aerial | Electrician supplied & installed | Location: Roof mounted while edge protection installed | Signal testing: not included |
| Switch Plates | Brand: Builder's range | Type: Slimline switch plate range with LED Location: internally throughout | Reference: |
| Gatehouse Intercom | Manufacturer: Builder's range Type: intercom and gate buzzer | Style: TBC Function: TBC | Finish: Manufacturer Colour: White |
| Gatehouse Gate lock | Type: Automatic, handset operated with key/manual override - 12V Height: 1800mm above FFL (check FFL height) | Style: TBC Locations: Kitchen, Master Bedroom Function: wired back to intercom handsets Quantity: Two (2x) handsets | Finish: Manufacturer Colour: Stainless Steel/Aluminium Finish (depends on availability) |
| External Gate lock | Type: Padlock | Location: Pre-painted steel gate return from fence to house |  |
| Exhaust Fans | Brand: Builder's range or similar | Type: 150mm (minimum) ducted to external wall Locations: Powder, Butlers Pantry (separate switch), Bath, Ensuite Shower Ceiling Quantity: Four (4x) off | Finish: Manufacturers Colour: White Note: Rangehood(s) ducted to external wall as well Exhaust fan size depends on ceiling timber location, more space, bigger exhaust up to 270mm diameter will be installed (all cost the same price) |
| Ceiling Fans | Brand: TBC | Model: Builder's range or similar Locations: Living, Dining, Master Bed, Bed 2, Bed 3, Bed 4, Alfresco, Deck (2x off) Quantity: Nine (9x) off Size: 132cm Remote: Not Included | Finish: Manufacturer Colour: White |
| LED strip lights | Product: LED strip light Brand: Builder's range (SMD LED) | Location: Directly under Bathroom & Ensuite Vanity units Size: length of vanity unit Capacity: 14.4V Mounting Profile: 16mm deep recess line strip mounting Notes: Includes diffuser to led strip aluminium channel. Note: transformer to go inside shaving cabinet | Finish: Diffuser Colour: White |
| Robe back of wall lights | Product: LED strip light | Location: WIR & Bedroom Robes Length: full robe length to illuminate length of each robe Function: illuminates back of wall so a user can easily see clothing | Finish: low profile channel with diffuser - no soldered LED only plug in low voltage transformers to be used. Colour: White light |
| LIGHTING & FIXTURES- GENERALLY |  |  |  |
| External Wall Light Points | Type: LED | Type: Up down lights Locations: Front elevation masonry fence piers (3x off), Garage column/pillar (1x off), Gatehouse Pier (1x off) Capacity: TBC | Finish: Manufacturer Colour: White |
| Garden Lights | Type: LED | Type: Weatherproof spike lights Locations: Fence planter boxes, front garden bed Capacity: TBC | Finish: Manufacturer Colour: Black |
| Flood Lights | Type: LED | Type: Flood lights Locations: as per plan Capacity: TBC | Finish: Manufacturer Colour: White |
| Weather proof power points | Builder's range (or similar) | Type: TBC Location: as per plan | Finish: Manufacturer Colour: White |
| Eaves light points | Type: LED | Type: Downlight from Builders Range Location: As per plan Style: Round Capacity:TBC | Finish: Manufacturer Colour: White |
| Ceiling Light Fittings - Generally - Externally | Type: LED | Type: Downlight from Builders Range Location: As per plan Style: Round Capacity:TBC | Finish: Manufacturer Colour: White |
| Ceiling Light Fittings - Generally - Internally | Type: LED | Type: Downlight from Builders Range Location: As per plan Style: Round Capacity: TBC | Finish: Manufacturer Colour: White |
| Kill Switch (Internal switch point) | Type: Switch plate to match range specified elsewhere | Location: Entry wall beside front entry door Note: all non essential lighting & power services connected to this point (excludes fridge & alarm locations & all external power points) |  |
| HYDRAULIC SERVICES |  |  |  |
| Pressure limiting device | Brand: Plumber supplied & installed | Type: 500KPA water pressure limiting device: Location: externally located by plumber at rough in |  |
| Gas Connection | Material: Metal Supplied by: Plumber/Gas Fitter | Type: Bayonet type Locations: Deck BBQ, Living Room Type: Threaded connection Locations: Upright Gas cooktop to kitchen, gas cooktop to butlers pantry | Finish Internal: Chrome plated where possible Finish External: Weather resistant finish where possible |
| Hose Cocks | Product: Internal quality Material: Steel | Location: all WC’s and internal water isolation points | Finish: Chrome plated |
| Garden Taps | Product: Brass garden tap Material: Brass | Type: 15mm female Locations: Porch, Garage nib wall, Dining External Wall, Living Room external wall corner, Upper level Deck, Bed 3 Balcony | Finish: Manufacturer finished |
| Vacuum Breakers | Product: External quality Material: Brass | Location: To all external garden taps | Finish: Manufacturer finished |
| Yard Gullies | Material: Metal grates, plastic structure Pit type: strip grates, linear | Size: linear gully grates (subject to stormwater line falls) | Note: leaf catchers not included |
| IO Caps | Product: External quality Brand: TBC | Type: Brass with plastic surround (no concrete surround) | Note: if located in solid surface finished area, like concrete or paving use IO with removable concrete shroud. Remove shroud and integrate into surface finish |
| Cold water tap to fridge space | Product: Internal quality Chrome plated | Type: Hose cock style Location: in cupboard beside fridge space. | Drill 38mm hole through cupboard in line with hose cock through to fridge space |
| Cold water tap to Alfresco | Product: External quality | Type: Brass | Finish: Manufacturer finished |
| Cold water tap to Deck | Product: External quality | Type: Brass | Finish: Manufacturer finished |
| Cold water tap to Bed 3 Balcony | Product: External quality | Type: Brass | Finish: Manufacturer finished |
| Cold water tap to dishwasher space | Product: Internal quality Chrome plated | Type: Hose cock style Location: in cupboard beside dishwasher space to Kitchen & Butler's Pantry | Drill 38mm hole through cupboard in line with hose cock through to dishwasher space |
| Hot & Cold water tap to washing machine space | Product: Internal quality Chrome plated | Type: Hose cock style Location: in cupboard beside washing machine space. | Drill 38mm hole through cupboard in line with hose cock through to washing machine space |
| INTERNALLY |  |  |  |
| LININGS |  |  |  |
| Wall & Ceiling linings (PB) | Brand: Builder's range | Size: 10mm | Finish: Painted Colour: see paint schedule below |
| Media Room Walls (PB2) | Brand: Builder's range | Size: 2x layers of 10mm plasterboard - 20mm thick overall Note: Door jambs wider to suit additional 10mm lining | Finish: Painted Colour: see paint schedule below |
| Alfresco & Deck Ceiling Linings (LIN.1) | Brand: Builder's range or similar Product: V-joint fibre cement or hardboard lining - TBC | Type: 150mm wide joints | Finish: Painted Colour: see paint schedule below |
| Wet Area Walls (LIN.2) | Brand: Builder's range Product: Fibre cement wet area lining board | Size: 8mm thick, set joints | Finish: Painted Colour: see paint schedule below |
| Cornice (COR1) | Product: Plasterboard cornice Manufacturer: Builder's range | Type: 90mm New York profile Locations: All areas other than MEDIA, KITCHEN, DINING, LIVING, ENTRY, HALL FROM ENTRY TO KITCHEN | Finish: Painted Colour: see paint schedule below |
| Cornice (COR2) | Product: Plasterboard cornice Manufacturer: Builder's range | Type: New York profile Locations: MEDIA, KITCHEN, DINING, LIVING, ENTRY, HALL FROM ENTRY TO KITCHEN | Finish: Painted Colour: see paint schedule below |
| Square Set Cornice (SQS1) | Material: Metal internals | Note: Square set finish to wall/ceiling interface Locations: Media bulkhead, Entry bulkhead, Living, Dining, Study Nook, Master Suite bulkhead, Picture Niche | Finish: Painted Colour: see paint schedule below |
| INTERNAL WOODWORK, TIMBER DOORS & FITTINGS |  |  |  |
| Skirtings | Brand: Builder's range Material: Timber (not mdf) | Profile: Moulded Size: 90mm x 19mm | Finish: Painted Colour: see paint schedule below |
| Architraves | Brand: Builder's range Material: Timber (not mdf) | Profile: DAR Square Dressed Size: 90mm x 19mm | Finish: Painted Colour: see paint schedule below |
| Header Architrave | Brand: Builder's range Material: Timber (not mdf) | Profile: Moulded Size: 90mm x 19mm Location: Across header with mitre return back into wall | Finish: Painted Colour: see paint selection below |
| Internal Doors (including Garage Access Door) | Brand: Builder's range Material: Timber | Type: Light Commercial Size: 2340mm Notes: 150mm rails & styles, polystyrene filled semi solid light commercial grade Hinges: Three (3) Face hole and striker backset: 54mm & 68mm backset Latch: Supplied with handle | Finish: Painted Colour: see paint schedule below Edges: Painted 3x coats all edges (sealer plus two top coats acrylic) Latch: pre-drilled & fitted at factory Latch Colour: Satin Chrome Striker: fitted onsite, not factory Striker Colour: Satin Chrome |
| Internal door frames | Brand: Builder's range | Type: Internal 2340mm primed pine Size: 112x19mm Door stops: 42x19mm pine | Note: Timber Finish: Painted Colour: see paint schedule below |
| Internal door hinges | Brand: Builder's range | Size: 3x off 100mm x 87mm hinges Type: Quick fit (no rebate of door required) Location: fitted to each hinged door, two at door top 150mm apart, one mid span, one bottom span Supplier: Hardware supplier | Hinge Screws: 38mm Colour: Chrome or Satin finish depending on availability |
| DOOR FURNITURE & ACCESSORIES |  |  |  |
| Passage Sets: Internal hinged doors | Brand: Builder's range | Location: Laundry, Store (lower), Linen/Store, Kitchen, Store (upper), WIR Model: Lever handle - 54mm backplate Keyed: Alike to aluminium windows & doors | Colour: Satin Chrome |
| Privacy Sets: Internal hinged doors to: | Brand: Builder's range | Location: Powder, Bed 3, Bed 4, WC, Bath, Bed 2, Ensuite, WC, Master Bed Model: Lever handle | Colour: Satin Chrome |
| Door Stops: Pole magnetic stops | Brand: Builder's range | Location :Laundry (2x off), Linen/Store, Kitchen, Entry, Bed 3, Bed 4, WC, Bath, Bed 2, Master Bed, WIR, Ensuite, Ensuite WC Model: Pole | Colour: Satin Chrome |
| Door Stops: Pole cushions | Brand: Builder's range | Size: 75mm Location: Store (lower), Powder, Store (upper) | Colour: Satin Chrome |
| INTERNAL STAIRS |  |  |  |
| Handrail & wall mounting brackets | Material: Timber | Profile: Breadtop or similar profile | Finish: Clear, matt painted finish (see paint selection below) Mounting brackets: round base, round profile generally |
| EXTERNAL STAIRS |  |  |  |
| Treads | Material: Aluminium | Size: 50mm thick, 900mm wide | Finish: Natural |
| Handrail & wall mounting brackets | Material: Timber | Profile: Breadtop or similar profile | Finish: Clear, matt painted finish (see paint selection below) Mounting brackets: round base, round profile generally |
| Stringers | Material: Steel |  | Finish: Galvanised |
| KITCHEN CABINETS |  |  |  |
| Cupboards | Manufacturer: Cabinetmaker constructed & installed Material: Laminated 12mm HMR chipboard | Location: under benchtops & overheads as per plan locations Type: One (1) intermediate shelf and a nest of four (4) drawers and a microwave provision. Height: 900mm high Width (generally): 600mm wide cupboards Cooker Cabinets Width: to be minimum 615mm deep ILO 610mm. Cooker to be placed on top of plinth (skirting runs past cabinets, past cooker space, cooker sits on top of plinth) | Colour: Lini Cava Finish: Matt Style: Textured Selection: Selected from Builder’s Range. |
| Pantry | Manufacturer: Cabinetmaker constructed & installed Material: Laminated 12mm HMR chipboard | Type: Laminated Pantry unit Size: 2400mm high Layout: Four (4) shelves and one (1) overhead cupboards | Colour: Lini Cava Finish: Smooth (not textured finish) Selection: Selected from Builder’s Range. |
| Fridge alcove height | Note: Clear height from concrete floor to underside of overhead cupboard. | Height: 1900mm | Note: Option to alter height to suit fridge available by Variation if requested. |
| Cupboards to ceiling with shadow line | Manufacturer: Cabinetmaker constructed & installed Material: Laminate to match overhead cupboards Note: Painted | Location: Entire kitchen overhead cupboards and above pantry unit. Laminated infill setback from overhead cabinets sitting against plasterboard bulkheads. | Colour: same as cupboard laminate Selection: Selected from Builder’s Range Colours Note: Light or dark colours require additional prep & finish coats |
| Door & Drawer Fronts | Manufacturer: Cabinetmaker | Edging Profile: Square edge door and drawer fronts Edging Type: PVC edging | Colour (front & back): Lini Cava Finish: Matt PVC Edge Colour: TBC PVC Edge Thickness: 2mm |
| Door & Drawer Handles | Manufacturer: Cabinetmaker | Type: Handles and knobs from builders range Style: TBC | Colour: same as L1005 Riverlinks Style: TBC |
| Door & Drawer Accessories | Brand: Builder's range Model: TBC | Location: All doors and drawers Type: Soft closers | Notes: |
| Benchtops | Product: Reconstituted stone Thickness: 20mm thick with 80mm thick edges | Edge thickness: 80mm Note: Edges mitre Note: Waterfall gable end to Kitchen island included (80mm thick) Note: Edges mitre finished | Colour: Snow-Fresh Concrete-Sleek Concrete-Raven-Oyster-Buttermilk-Wild Rocks-Atlantic Salt- Nougat-Clamshell (Note: one colour throughout allowed) Sealing: Not Included or required Note: Benchtop height is 950mm above slab level |
| Kickboard | Manufacturer: Cabinetmaker constructed & Installed Material: 12mm HMR chipboard | Location: Entire kitchen cupboard units | Colour: Lini Cava Finish: Matt Selection: Selected from Builder’s Range. |
| KITCHEN ITEMS |  |  |  |
| Kitchen Sink | Brand: Builder's range | Type: Undermount Size: Large single bowl - 810mm x 505mm x 250mm | Colour: Black Accessories: No additional included |
| Kitchen Sink Accessory | Brand: Builder's range | Type: Folding over-sink dish rack | Colour: Stainless Steel |
| Kitchen Sink Mixer | Brand: Builder's range | Type: Pull Down Sink Mixer Style: Pull-down spring mixer Rating: 6.5L/min | Colour: Chrome Plated |
| Cooktop & Oven: | Brand: Builder's range | Type: Upright Cooktop Dual Fuel Size: 900mm | Colour: Stainless Steel |
| Microwave | Brand: Builder's range |  | Colour: Stainless Steel |
| Microwave Trim Kit | Brand: Builder's range |  | Colour: Stainless Steel |
| Rangehood | Brand: Builder's range | Type: Undermount Rangehood Size: 96.5cm | Colour: Stainless Steel Note: ducted through to external wall vent (through wall) |
| Dishwasher | Brand: Builder's range | Type: Built In Size: 60cm | Colour: Stainless Steel |
| BUTLERS PANTRY CABINETS |  |  |  |
| Cabinets - open shelves | Manufacturer: Cabinetmaker constructed & installed Material: Laminated 12mm HMR chipboard with PVC edge | Location: adjacent to pantry access door, entire elevation A wall of butlers pantry Type: Seven (7) parcel shelves fixed inside cabinet spaced evenly across vertical height of cabinet. Edging Type: PVC edging Size: 450mm deep shelves. | Colour: White Finish: Laminated PVC Edge Colour: White PVC Edge Thickness: 2mm |
| Benchtops | Product: Category 2 reconstituted stone Thickness: 20mm thick | Edge thickness: 80mm Note: Edges mitred | Colour: Snow-Fresh Concrete-Sleek Concrete-Raven-Oyster-Buttermilk-Wild Rocks-Atlantic Salt- Nougat-Clamshell (Note: one colour throughout allowed) Selection: Sealing: Not Included or required Note: Benchtop height is 950mm above slab level |
| Cupboards | Manufacturer: Cabinetmaker constructed & installed Material: Laminated 12mm HMR chipboard | Location: under benchtops & overheads above as per plan location Type: One (1) intermediate shelf and two (2) nests of four (4) drawers and microwave provision. Size: 600mm wide cupboards. | Colour: Lini Cava Finish: Matt Selection: Selected from Builder’s Range. |
| Cupboards to ceiling with shadow line | Manufacturer: Cabinetmaker constructed & installed Material: Laminate to match overhead cupboards Note: Painted | Location: Entire kitchen overhead cupboards and above pantry unit. Laminated infill setback from overhead cabinets sitting against plasterboard bulkheads. | Colour: same as cupboard laminate Selection: Selected from Builder’s Range Colours Note: Light or dark colours require additional prep & finish coats |
| Door & Drawer Fronts | Manufacturer: Cabinetmaker constructed & Installed Material: Laminated 12mm HMR chipboard | Edging Profile: Square edge door and drawer fronts Edging Type: PVC edging | Colour (front & back): Cava Lini Finish: Matt PVC Edge Colour: TBC PVC Edge Thickness: 2mm |
| Door & Drawer Handles | Manufacturer: Cabinetmaker | Type: Handles and knobs from builders range Style: TBC | Colour: same as L1005 Riverlinks Style: TBC |
| Door & Drawer Accessories | Brand: Builder's range Model: TBC | Location: All doors and drawers Type: Soft closers |  |
| Drawer Accessories | Brand: Builder's range | Location: Two drawers in kitchen and two drawers in butlers pantry allow for stainless steel cutlery drawer inserts | Colour: Stainless steel |
| Drawer sides | Brand: Builder's range | Type: Double-wall metal drawer system | Colour: White |
| Kickboard | Manufacturer:: Cabinetmaker constructed & Installed Material: 12mm HMR chipboard | Location: Entire kitchen cupboard units | Colour: Lini Cava Finish: Matt Selection: Selected from Builder’s Range. |
| BUTLERS PANTRY ITEMS |  |  |  |
| Sink | Brand: Builder's range | Type: Undermount Size: Large single bowl - 810mm x 505mm x 250mm | Colour: Black Accessories: No additional included |
| Kitchen Sink Mixer | Brand: Builder's range | Type: Pull Down Sink Mixer Style: Pull-down spring mixer Rating: 6.5L/min | Colour: Chrome Plated |
| Dishwasher | Brand: Builder's range | Type: Freestanding Size: 60cm | Colour: Stainless Steel |
| LAUNDRY CABINETS |  |  |  |
| Benchtops | Product: Reconstituted stone Thickness: 20mm thick | Edge thickness: 80mm Note: Edges mitre finished | Colour: Snow-Fresh Concrete-Sleek Concrete-Raven-Oyster-Buttermilk-Wild Rocks-Atlantic Salt- Nougat-Clamshell (Note: one colour throughout allowed) Sealing: Not Included or required Note: Benchtop height is 950mm above slab level |
| Cupboards | Manufacturer: Cabinetmaker constructed & installed Material: Laminated 12mm HMR chipboard | Location: under benchtops & overheads above as per plan location Type: One (1) intermediate shelf Size: 600mm wide cupboards. Splashback Height: 600mm from Benchtop to under​side of overheads | Colour: Lini Cava Finish: Matt Selection: Selected from Builder’s Range. |
| Cupboards to ceiling with shadow line | Manufacturer: Cabinetmaker constructed & installed Material: Laminate to match overhead cupboards Note: Painted | Location: Entire kitchen overhead cupboards and above pantry unit. Laminated infill setback from overhead cabinets sitting against plasterboard bulkheads. | Colour: same as cupboard laminate Selection: Selected from Builder’s Range Colours Note: Light or dark colours require additional prep & finish coats |
| Door & Drawer Fronts | Manufacturer: Cabinetmaker constructed & Installed Material: Laminated 12mm HMR chipboard | Edging Profile: Square edge door and drawer fronts Edging Type: PVC edging | Colour (front & back): Cava Lini Finish: Matt PVC Edge Colour: TBC PVC Edge Thickness: 2mm |
| Door & Drawer Handles | Manufacturer: Cabinetmaker | Type: Integrated finger pulls Style: Rebated or recessed top edge - TBC | Colour: Same as door & drawer fronts |
| Door & Drawer Accessories | Brand: Builder's range Model: TBC | Location: All doors and drawers Type: Soft closers |  |
| Kickboard | Manufacturer:: Cabinetmaker constructed & Installed Material: 12mm HMR chipboard | Location: Entire cupboard units | Colour: Cava Lini Finish: Matt Selection: Selected from Builder’s Range. |
| Door & Drawer Accessories | Brand: Builder's range Model: TBC | Location: All doors and drawers Type: Soft closers |  |
| Drawer sides | Brand: Builder's range | Type: Double-wall metal drawer system | Colour: White |
| LAUNDRY ITEMS |  |  |  |
| Kitchen Sink | Brand: Builder's range | Type: Undermount Size: Large single bowl - 805mm x 505mm x 250mm | Colour: Black Accessories: No additional included |
| Mixer | Brand: Builder's range | Type: To Be Confirmed Model: TBA | Colour: Chrome Note: Bench mounted mixer |
| Exhaust Vent | Brand: Hardware supplier Product: Clothes dryer through-wall vent kit | Location: 300mm above ground level Size: 100mm dia external vent (fixed to cladding) and ducted into bench cabinet to dryer space for future connection | Colour: Natural Aluminium Note: duct to extend into cabinet. Duct end to be capped off and duct to be completely sealed with pipe shroud to prevent vermin & pest ingress through cavity |
| Floor Waste - Dry Waste | Brand: Builder's range Product: Square grate floor waste | Type: Floor waste Shape: Square Size: 50mm x 50mm | Colour: Stainless Steel |
| ALFRESCO ITEMS |  |  |  |
| BBQ Plate | Brand: Builder's range, 900mm integrated BBQ | Type: Integrated Size: 900mm | Colour: Stainless Steel |
| Rangehood | Brand: Builder's range | Type: Undermount Rangehood Size: 96.5cm | Colour: Stainless Steel Note: ducted through to external wall vent (through wall) |
| Benchtops | Product: Reconstituted stone Thickness: 20mm thick | Edge thickness: 80mm Note: Edges mitre finished | Colour: Snow-Fresh Concrete-Sleek Concrete-Raven-Oyster-Buttermilk-Wild Rocks-Atlantic Salt- Nougat-Clamshell (Note: one colour throughout allowed) Sealing: Not Included or required Note: Benchtop height is 950mm above slab level |
| Cupboards | Manufacturer: Cabinetmaker constructed & installed Material: External waterproof quality materials Location: Wall mounted unit with 300mm clearance from FFL to underside of unit | Location: under benchtops & overheads to same length as benchtops overalls Type: One (1) intermediate shelf Size: 600mm wide cupboards, Pot drawers to remainder Splashback Height: 800mm from Benchtop to under​side of overhead | Colour: Lini Cava Finish: Laminated Selection: Selected from Builder’s Range. |
| Cupboards to bulkhead with shadow line | Manufacturer: Cabinetmaker constructed & installed Material: Laminate to match overhead cupboards Note: Painted Length: 2400mm with side returns | Location: Entire overhead cupboards. Laminated infill setback from overhead cabinets sitting against plasterboard bulkheads. | Colour: same as cupboard laminate Selection: Selected from Builder’s Range Colours Note: Light or dark colours require additional prep & finish coats |
| Door & Drawer Fronts | Manufacturer: Cabinetmaker constructed & Installed Material: External waterproof quality materials | Edging Profile: Square edge door and drawer fronts Edging Type: PVC edging | Colour (front & back): TBC Finish: Laminated PVC Edge Colour: TBC PVC Edge Thickness: 2mm |
| Door & Drawer Handles | Manufacturer: Cabinetmaker | Type: Integrated finger pulls Style: Rebated or recessed top edge - TBC | Colour: Same as door & drawer fronts |
| Door & Drawer Accessories | Brand: Builder's range Model: TBC | Location: All doors and drawers Type: Soft closers |  |
| Drawer Inserts | Brand: Builder's range Model: TBC | Location: Two Pot Drawers Type: Stainless steel drawer dividers to suit drawer width | Quantity: 2x pot drawers allowance |
| Drawer sides | Brand: Builder's range | Type: Double-wall metal drawer system | Colour: White |
| POWDER ROOM ITEMS |  |  |  |
| Toilet Suite | Brand: Builder's range | Type: Close coupled toilet suite Model: Back-to-wall (BTW) Inlet Position: top mounted inlet Seat: soft close | Colour: White |
| Vanity Basin | Brand: Builder's range | Model: Wall-hung basin Size: 450mm x 250mm Type: Wall mounted Note: 1 tap hole | Colour: White |
| Vanity Basin Towel Rail | Brand: Builder's range Style: Matching basin range | Location: front vanity mounted Size: 365mm | Colour: Stainless Steel |
| Vanity waste | Brand: Builder's range | Product: Pop up style waste Size: 32/40 | Colour: Chrome |
| Vanity Trap | Brand: Builder's range | Model: Matching chrome suite Size: 40mm round bottle trap | Colour: Chrome |
| Floor Waste | Brand: Builder's range Product: Square grate floor waste | Type: Floor waste Shape: Square Size: 100mm x 100mm | Colour: Stainless Steel |
| Towel Rail | Brand: Builder's range | Model: Front basin towel rail Size: 365mm front basin towel rail | Colour: Stainless Steel finish |
| Tapware | Brand: Builder's range | Model: Matching chrome suite Type: Basin Mixer | Colour: Chrome |
| WC Roll Holder | Brand: Builder's range | Model: Matching chrome suite Location: Wall mounted right hand of seated WC position | Colour: Chrome |
| WC Toilet Brush Holder | Brand: Builder's range | Model: Matching chrome suite Type: Location: Wall mounted beside WC position | Colour: Chrome |
| Mirror | Brand: Supplier Product: 4mm arrised edge | Location: mounted on top of splashback tiles Type: Wall mounted Trim: Micro frame to entire mirror | Colour: silver backed mirror |
| Towel rail | Brand: Builder's range Product: Single towel rail Length: 900mm | Location: Bathroom Type: Wall mounted Location: centrally across wall space between bath edge and door jamb 1 @ 1000mm CL above FFL, 1 @ 1850mm above Quantity: Two (x2) off | Colour: Chrome |
| Soap dish | Brand: Builder's range Product: Wall-mounted soap basket | Location: shower wall Type: Wall mounted @ 1500 above FFL Quantity: One (1x) off | Colour: Chrome |
| Robe hook | Product: Builder's range Brand: | Location: In front of shower, wall mounted @ 1800mm above FFL Type: Wall mounted Quantity: One (x1) off | Colour: Chrome |
| Shower tapware | Brand: Builder's range | Location: Shower Type: Wall top assembly Notes: | Colour: Chrome |
| Shower rain head | Brand: Builder's range Product: | Location: Shower Type: Ceiling mounted Size: 300mm | Colour: Chrome |
| Shower arm | Brand: Builder's range | Location: Shower Size: 300mm | Colour: Chrome |
| Shower Floor Waste | Brand: Builder's range Product: Shower Grate | Model: 65mm wide channel grate Size: 1800mm | Colour: Stainless Steel Reference: |
| Shower screen | Product: Semi Frameless Glass Brand: TBC | Location: Bathroom Type: Wall mounted Notes: Door Handle knob pull | Fitting Colour: Chrome |
| WC ITEMS |  |  |  |
| Toilet Suite | Brand: Builder's range | Type: Close coupled toilet suite Model: Back-to-wall (BTW) Inlet Position: top mounted inlet Seat: soft close | Colour: White |
| WC Roll Holder | Brand: Builder's range | Model: Matching chrome suite Location: Wall mounted right hand of seated WC position | Colour: Chrome |
| WC Toilet Brush Holder | Brand: Builder's range | Model: Matching chrome suite Type: Location: Wall mounted beside WC position | Colour: Chrome |
| ENSUITE ITEMS |  |  |  |
| Bathtub | Product: Freestanding bath Brand: Builder's range | Location: Bathroom Size: 1650mm Notes: Acrylic bath | Colour: White |
| Bath tapware | Brand: Builder's range | Location: Bathroom Type: Wall top assembly Notes: | Colour: Chrome |
| Bath spout | Brand: Builder's range | Type: Floor mounted Model: Matching chrome suite | Colour: Chrome |
| Bath waste | Brand: Builder's range | Product: Pop up style waste Size: 32/40 Note: waste penetration through floor clearance? | Colour: Chrome |
| Floor Waste | Brand: Builder's range Product: Square grate floor waste | Type: Floor waste Shape: Square Size: 100mm x 100mm | Colour: Stainless Steel |
| Vanity tapware set | Brand: Builder's range Note: wall mounted | Model: Matching chrome suite Size: 200 Type: Curved wall basin | Colour: Chrome |
| Shaving Cabinet | Brand: Cabinetmaker constructed | Location: Bathroom Type: Wall mounted Size: 1400mm x 400mm high x 150mm deep Note: recessed into wall | Colour: Standard silver backed mirror Frame Colour: TBC Note: Three (3) hinges to all doors |
| Vanity Basin | Brand: Builder's range | Location: Bathroom Type: Undermount Notes: One (1x) basin | Colour: White |
| Benchtop | Product: reconstituted stone top with “thick edge”, underslung vanity | Top Thickness: 20mm thick Edge type: mitred Benchtop width: 470mm OA | Colour: Snow-Fresh Concrete-Sleek Concrete-Raven-Oyster-Buttermilk-Wild Rocks-Atlantic Salt- Nougat-Clamshell (Note: one colour throughout allowed) Note: Benchtop height is 950mm above slab level |
| Cabinets | Manufacturer: Cabinetmaker constructed & installed Material: Laminated 12mm HMR chipboard Mounting: Wall mounted Note: 19mm rebate under drawer length of cabinet to allow for LED strip light installation. Transformer BOX inside vanity door cabinet space. Small enclosure inside cabinet with laminated face panel (removable to maintenance) | Type: two (2) drawer & four (4) doors Location: position as per plan Size: 450mm deep cabinet. Length: 2400mm (two cabinets mirror image with top full span across both units) Note: keep at least 200mm between bottom edge of cabinet and floor level (for cleaning) | Colour: TBC (one colour allowed) Finish: Laminated Selection: Selected from Builder’s Range. |
| Drawer fronts | Manufacturer: Cabinetmaker constructed & Installed Material: 12mm HMR chipboard | Edging Profile: Square edge door and drawer fronts Edging Type: PVC edging | Colour (front & back): TBC Finish: Laminated PVC Edge Colour: TBC PVC Edge Thickness: 2mm Selection: Selected from Builder’s Range. |
| Drawer handles | Manufacturer: Cabinetmaker | Type: Integrated finger pulls Style: Rebated or recessed top edge - TBC | Colour: Same as drawer fronts |
| Door & Drawer Accessories | Brand: Builder's range Model: TBC | Location: All doors and drawers Type: Soft closers |  |
| Drawer sides | Brand: Builder's range | Type: Double-wall metal drawer system | Colour: White |
| Vanity waste | Brand: Builder's range | Product: Pop up style waste Size: 32/40 Quantity: 2x off | Colour: Chrome |
| Towel rail | Brand: Builder's range Product: Single towel rail Length: 900mm | Location: Bathroom Type: Wall mounted Location: centrally across wall space between bath edge and door jamb 1 @ 1000mm CL above FFL, 1 @ 1850mm above Quantity: Two (x2) off | Colour: Chrome |
| Soap dish | Brand: Builder's range Product: Wall-mounted soap basket | Location: shower wall Type: Wall mounted @ 1500 above FFL Quantity: One (1x) off | Colour: Chrome |
| Robe hook | Product: Builder's range Brand: | Location: In front of shower & beside bath adjacent to double towel rail,, wall mounted @ 1800mm above FFL Type: Wall mounted Quantity: Two (x2) off | Colour: Chrome |
| Toilet Suite | Brand: Builder's range | Type: Close coupled toilet suite Model: Back-to-wall (BTW) Inlet Position: top mounted inlet Seat: soft close | Colour: White |
| WC Roll Holder | Brand: Builder's range | Model: Matching chrome suite Location: Wall mounted right hand of seated WC position | Colour: Chrome |
| WC Toilet Brush Holder | Brand: Builder's range | Model: Matching chrome suite Type: Location: Wall mounted beside WC position | Colour: Chrome |
| Shower tapware | Brand: Builder's range | Location: Shower Type: Wall top assembly Notes: | Colour: Chrome |
| Shower rain head | Brand: Builder's range Product: | Location: Shower Type: Ceiling mounted Size: 300mm | Colour: Chrome |
| Shower arm | Brand: Builder's range | Location: Shower Size: 300mm | Colour: Chrome |
| Shower Floor Waste | Brand: Builder's range Product: Shower Grate | Model: 65mm wide channel grate Size: 1800mm | Colour: Stainless Steel Reference: |
| Shower screen | Product: Semi Frameless Glass Brand: TBC | Location: Bathroom Type: Wall mounted Notes: Door Handle knob pull | Fitting Colour: Chrome |
| BATHROOM ITEMS |  |  |  |
| Bathtub | Product: Freestanding bath Brand: Builder's range | Location: Bathroom Size: 1500mm Notes: Acrylic bath | Colour: White |
| Bath tapware | Brand: Builder's range | Location: Bathroom Type: Wall top assembly Notes: | Colour: Chrome |
| Bath Spout | Brand: Builder's range | Type: Floor mounted Model: Matching chrome suite | Colour: Chrome |
| Bath waste | Brand: Builder's range | Product: Pop up style waste Size: 32/40 Note: waste penetration through floor, clearance? | Colour: Chrome |
| Floor Waste | Brand: Builder's range Product: Square grate floor waste | Type: Floor waste Shape: Square Size: 100mm x 100mm | Colour: Stainless Steel |
| Vanity tapware set | Brand: Builder's range Note: wall mounted | Model: Matching chrome suite Size: 200 Type: Curved wall basin | Colour: Chrome |
| Vanity Basin | Brand: Builder's range Style: Matching basin range | Location: Bathroom Type: Wall Mounted Size: 900mm wide Code: Notes: No tap holes (wall top assembly) | Colour: White Reference: |
| Vanity Basin Towel Rail | Brand: Builder's range Style: Matching basin range | Location: front vanity mounted Size: 498mm | Colour: Stainless Steel |
| Vanity waste | Brand: Builder's range | Product: Pop up style waste Size: 32/40 | Colour: Chrome |
| Vanity Trap | Brand: Builder's range | Model: Matching chrome suite Size: 40mm round bottle trap | Colour: Chrome |
| Towel rail | Brand: Builder's range Product: Single towel rail Length: 900mm | Location: Bathroom Type: Wall mounted Location: centrally across wall space between bath edge and door jamb 1 @ 1000mm CL above FFL, 1 @ 1850mm above Quantity: Two (x2) off | Colour: Chrome |
| Soap dish | Brand: Builder's range Product: Wall-mounted soap basket | Location: shower wall Type: Wall mounted @ 1500 above FFL Quantity: One (1x) off | Colour: Chrome |
| Robe hook | Product: Builder's range Brand: | Location: In front of shower & beside bath adjacent to double towel rail,, wall mounted @ 1800mm above FFL Type: Wall mounted Quantity: Two (x2) off | Colour: Chrome |
| Shower tapware | Brand: Builder's range | Location: Shower Type: Wall top assembly Notes: | Colour: Chrome |
| Shower rain head | Brand: Builder's range Product: | Location: Shower Type: Ceiling mounted Size: 300mm | Colour: Chrome |
| Shower arm | Brand: Builder's range | Location: Shower Size: 300mm | Colour: Chrome |
| Shower Floor Waste | Brand: Builder's range Product: Shower Grate | Model: 65mm wide channel grate Size: 900mm | Colour: Stainless Steel Reference: |
| Shower screen | Product: Semi Frameless Brand: TBC | Location: Bathroom Type: Wall mounted Notes: Door Handle knob pull | Fitting Colour: Chrome |
| Shaving Cabinet | Brand: Builder's range | Location: Bathroom Type: Wall mounted Size: 900mm x 400mm high x 150mm deep Note: face fixed to wall (not recessed) | Colour: Standard silver backed mirror Cabinet Colour: TBC Note: Three (3) hinges to all doors |
| BEDROOM 4 ENSUITE |  |  |  |
| Floor Waste | Brand: Builder's range Product: Square grate floor waste | Type: Floor waste Shape: Square Size: 100mm x 100mm | Colour: Stainless Steel |
| Vanity tapware set | Brand: Builder's range Note: wall mounted | Model: Matching chrome suite Size: 200 Type: Curved wall basin | Colour: Chrome |
| Vanity Basin | Brand: Builder's range Style: Matching basin range | Location: Bathroom Type: Wall Mounted Size: 600mm wide Code: Notes: No tap holes (wall top assembly) | Colour: White Reference: |
| Vanity Basin Towel Rail | Brand: Builder's range Style: Matching basin range | Location: front vanity mounted Size: 498mm | Colour: Stainless Steel |
| Vanity waste | Brand: Builder's range | Product: Pop up style waste Size: 32/40 | Colour: Chrome |
| Vanity Trap | Brand: Builder's range | Model: Matching chrome suite Size: 40mm round bottle trap | Colour: Chrome |
| Towel rail | Brand: Builder's range Product: Single towel rail Length: 900mm | Location: Bathroom Type: Wall mounted Location: centrally across wall space between bath edge and door jamb 1 @ 1000mm CL above FFL, 1 @ 1850mm above Quantity: Two (x2) off | Colour: Chrome |
| Soap dish | Brand: Builder's range Product: Wall-mounted soap basket | Location: shower wall Type: Wall mounted @ 1500 above FFL Quantity: One (1x) off | Colour: Chrome |
| Robe hook | Product: Builder's range Brand: | Location: In front of shower & beside bath adjacent to double towel rail,, wall mounted @ 1800mm above FFL Type: Wall mounted Quantity: Two (x2) off | Colour: Chrome |
| Shower tapware | Brand: Builder's range | Location: Shower Type: Wall top assembly Notes: | Colour: Chrome |
| Shower rain head | Brand: Builder's range Product: | Location: Shower Type: Ceiling mounted Size: 300mm | Colour: Chrome |
| Shower arm | Brand: Builder's range | Location: Shower Size: 300mm | Colour: Chrome |
| Shower Floor Waste | Brand: Builder's range Product: Shower Grate | Model: 65mm wide channel grate Size: 900mm | Colour: Stainless Steel Reference: |
| Shower screen | Product: Semi Frameless Brand: TBC | Location: Bathroom Type: Wall mounted Notes: Door Handle knob pull | Fitting Colour: Chrome |
| Shaving Cabinet | Brand: Builder's range | Location: Bathroom Type: Wall mounted Size: 900mm x 400mm high x 150mm deep Note: face fixed to wall (not recessed) | Colour: Standard silver backed mirror Cabinet Colour: TBC Note: Three (3) hinges to all doors |
| BEDROOM 3 ENSUITE |  |  |  |
| Floor Waste | Brand: Builder's range Product: Square grate floor waste | Type: Floor waste Shape: Square Size: 100mm x 100mm | Colour: Stainless Steel |
| Vanity tapware set | Brand: Builder's range Note: wall mounted | Model: Matching chrome suite Size: 200 Type: Curved wall basin | Colour: Chrome |
| Vanity Basin | Brand: Builder's range Style: Matching basin range | Location: Bathroom Type: Wall Mounted Size: 600mm wide Code: Notes: No tap holes (wall top assembly) | Colour: White Reference: |
| Vanity Basin Towel Rail | Brand: Builder's range Style: Matching basin range | Location: front vanity mounted Size: 498mm | Colour: Stainless Steel |
| Vanity waste | Brand: Builder's range | Product: Pop up style waste Size: 32/40 | Colour: Chrome |
| Vanity Trap | Brand: Builder's range | Model: Matching chrome suite Size: 40mm round bottle trap | Colour: Chrome |
| Towel rail | Brand: Builder's range Product: Single towel rail Length: 900mm | Location: Bathroom Type: Wall mounted Location: centrally across wall space between bath edge and door jamb 1 @ 1000mm CL above FFL, 1 @ 1850mm above Quantity: Two (x2) off | Colour: Chrome |
| Soap dish | Brand: Builder's range Product: Wall-mounted soap basket | Location: shower wall Type: Wall mounted @ 1500 above FFL Quantity: One (1x) off | Colour: Chrome |
| Robe hook | Product: Builder's range Brand: | Location: In front of shower & beside bath adjacent to double towel rail,, wall mounted @ 1800mm above FFL Type: Wall mounted Quantity: Two (x2) off | Colour: Chrome |
| Shower tapware | Brand: Builder's range | Location: Shower Type: Wall top assembly Notes: | Colour: Chrome |
| Shower rain head | Brand: Builder's range Product: | Location: Shower Type: Ceiling mounted Size: 300mm | Colour: Chrome |
| Shower arm | Brand: Builder's range | Location: Shower Size: 300mm | Colour: Chrome |
| Shower Floor Waste | Brand: Builder's range Product: Shower Grate | Model: 65mm wide channel grate Size: 900mm | Colour: Stainless Steel Reference: |
| Shower screen | Product: Semi Frameless Brand: TBC | Location: Bathroom Type: Wall mounted Notes: Door Handle knob pull | Fitting Colour: Chrome |
| Shaving Cabinet | Brand: Builder's range | Location: Bathroom Type: Wall mounted Size: 900mm x 400mm high x 150mm deep Note: face fixed to wall (not recessed) | Colour: Standard silver backed mirror Cabinet Colour: TBC Note: Three (3) hinges to all doors |
| MIRRORS - BEDROOMS & STUDY NOOK |  |  |  |
| Arrised edge mirror - wall mounted - master bedroom | Brand: Builder's range Product: 4mm arrised edge mirror | Location: wall, alcove adjacent to WIR & Ensuite door entry Size: 2100mm H x 600mm W | Notes: mounted after painting prior to handover. Stuck directly to plasterboard wall Note 2: Installed by builder |
| Study Nook | Brand: Builder's range Product: 4mm arrised edge mirror | Location: inside study nook as “splashback” above benchtop Size: length x width of nook splashback ## Check height and width to make sure is possible in 1 piece## | Notes: mounted after painting prior to handover. Stuck directly to plasterboard wall Note: painting not required behind mirror location Note 2: Installed by builder |
| ROBES - BEDROOMS |  |  |  |
| Cabinets | Manufacturer: Robe supplier, constructed & installed Material: Laminated 12mm HMR chipboard | Type: one (1) parcel shelf and a nest of four (4) drawers with remainder of robe length divided into ⅓ full hanging space with rail, ⅔ double hanging (equal vertical division) Shoe Shelf: kickboard and 150mm high by 600mm shoe shelf to run entire length of robe between drawer nests. Size: 600mm deep cupboards. | Colour: TBC (one colour allowed) Finish: Laminated Selection: Selected from Builder’s Range. |
| Drawer fronts | Manufacturer: Robe supplier, constructed & Installed Material: 12mm, Laminated finish | Edging Profile: Square edge door and drawer fronts Edging Type: PVC edging | Colour (front & back): TBC Finish: Laminated PVC Edge Colour: TBC PVC Edge Thickness: 2mm Selection: Selected from Builder’s Range. |
| Hanging Rail | Manufacturer: Robe supplier | Location: within robes to Bed 2, Bed 3, Bed 4 Material: Metal End Caps: Plastic Shape: Rectangular | Finish: Manufacturer Rail Colour: Chrome End Cap Colour: White Note: Intermediate hangers installed @ 600mm centres to rails over 1000mm in length (minimum) |
| WIR |  |  |  |
| Cabinets | Manufacturer: Robe supplier constructed & installed Material: Laminated 12mm HMR chipboard | Type: One (1) parcel shelf and two (2) nest of four (4) drawers with remainder of robe length divided into ⅓ full hanging space with rail, ⅔ double hanging (equal vertical division) Shoe Shelf: Kickboard and 150mm high by 600mm shoe shelf to run entire length of robe between drawer nests. Size: 600mm deep cupboards. | Colour: TBC (one colour allowed) Finish: Laminated Selection: Selected from Builder’s Range. |
| Drawer fronts | Manufacturer: Robe supplier, constructed & Installed Material: 12mm HMR chipboard | Edging Profile: Square edge door and drawer fronts Edging Type: PVC edging | Colour (front & back): TBC Finish: Laminated PVC Edge Colour: TBC PVC Edge Thickness: 2mm Selection: Selected from Builder’s Range. |
| Door & drawer handles | Manufacturer: Robe supplier | Type: Integrated finger pulls Style: Rebated or recessed top edge - TBC | Colour: Same as drawer fronts |
| Door & drawer accessories | Brand: Builder's range Model: TBC | Location: All doors and drawers inside robes Type: Soft closers | Colour: White Finish: Metal |
| Hanging Rail | Manufacturer: Robe supplier | Location: within robes to Bed 2, Bed 3, Bed 4 Material: Metal End Caps: Plastic Shape: Rectangular | Finish: Manufacturer Rail Colour: Chrome End Cap Colour: White Note: Intermediate hangers installed @ 600mm centres to rails over 1000mm in length (minimum) |
| STORES & LINEN CUPBOARDS |  |  |  |
| Cabinets | Manufacturer: Robe supplier, constructed & installed Material: Laminated 12mm HMR chipboard | Location: Linen/Store, Store beside Study Nook, Store adjacent to Master Suite, Type: Five (5) parcel shelves fixed inside cabinet spaced evenly across vertical height of cabinet. Size: 600mm deep cupboards. | Colour: TBC (one colour allowed) Finish: Laminated Selection: Selected from Builder’s Range. |
| STUDY NOOK |  |  |  |
| Benchtop | Product: Stained plywood C/D Grade Brand: Builder supplied & installed | Top Thickness: 32mm thick Profile: Square edge Benchtop width: 900mm - TBC | Colour: Grey/Brown - steel burnish wash Selection: Selected from Builder’s Range. Note: Benchtop height is 950mm above slab level |
| Cabinets | Manufacturer: Cabinetmaker constructed & installed Material: Laminated 12mm HMR chipboard | Type: Two (2) nest of four (4) drawers Location: position as per plan Kickboard:150mm high. Size: 600mm deep cabinet. | Colour: TBC (one colour allowed) Finish: Laminated Selection: Selected from Builder’s Range. |
| Drawer fronts | Manufacturer: Cabinetmaker constructed & Installed Material: 12mm HMR chipboard | Edging Profile: Square edge door and drawer fronts Edging Type: PVC edging | Colour (front & back): TBC Finish: Laminated PVC Edge Colour: TBC PVC Edge Thickness: 2mm Selection: Selected from Builder’s Range. |
| Drawer handles | Manufacturer: Cabinetmaker | Type: Integrated finger pulls Style: Rebated or recessed top edge - TBC | Colour: Same as drawer fronts |
| SPLASHBACK, CERAMIC TILING, TRIMS & CAULKING |  |  |  |
| Kitchen Splashback | Product: Ceramic Tiles | Type: 600mm x 600mm Code: TBC | Tile Colour: Grey Colour - TBC Grout By: Builder's range Grout Colour: Grey grout colour - with mould inhibitor |
| Butlers Pantry Splashback | Product: Ceramic Tiles | Type: 600mm x 600mm to floors Code: TBC | Tile Colour: Grey Colour - TBC Grout By: Builder's range Grout Colour: Grey grout colour - with mould inhibitor |
| Tile Trims | Brand: TBC | Type: square edge metal trim Locations: where tile biscuit exposed fit trim. | Colour: Chrome/raw finish |
| Bathroom Floor Tiles | Product: Ceramic tile Brand: | Type: 600mm x 600mm to floors Code: TBC | Tile Colour: Grey Colour - TBC Grout By: Builder's range Grout Colour: Grey grout colour - with mould inhibitor |
| Bathroom Wall Tiles | Product: Ceramic Tile Brand: | Type: 600mm x 600mm Code: TBC Note: Full height tiling to all walls | Tile Colour: Satin White - TBC Grout By: Builder's range Grout Colour: Black to floors & walls |
| Ensuite Floor Tiles | Product: Ceramic Tile Brand: TBC | Type: 600mm x 600mm to floors Code: TBC | Tile Colour: Grey Colour - TBC Grout By: Builder's range Grout Colour: Grey grout colour - with mould inhibitor |
| Ensuite Wall Tiles | Product: Ceramic Tile Brand: | Type: 600mm x 600mm Code: TBC Note: Full height tiling to all walls | Tile Colour: Satin White Grout By: Builder's range Grout Colour: Black to floors & walls |
| Laundry Floor Tiles | Product: Ceramic Tile Brand: | Type: 600mm x 600mm Code: TBC | Tile Colour: Grey Colour - TBC Grout By: Builder's range Grout Colour: Grey grout colour - with mould inhibitor |
| Laundry Wall Tiles | Product: Ceramic Tile Brand: | Type: 600mm x 600mm Code: TBC Note: 1200mm height tiling to all walls, splashback to benchtop fully tiled under overhead cupboards | Tile Colour: Satin White Grout By: Builder's range Grout Colour: Black to floors & walls |
| Powder Floor Tiles | Product: Ceramic Tile Brand: | Type: 450mm x 450mm Code: TBC | Tile Colour: Grey Colour - TBC Grout By: Builder's range Grout Colour: Grey grout colour - with mould inhibitor |
| Powder Wall Tiles | Product: Ceramic Tile Brand: | Type: 450mm x 450mm Code: TBC Note: Full height tiling to all walls | Tile Colour: Satin White Grout By: Builder's range Grout Colour: Black to floors & walls |
| WC Floor Tiles | Product: Ceramic Tile Brand: | Type: 450mm x 450mm Code: TBC Note: full height tiling to all walls | Tile Colour: Grey Colour - TBC Grout By: Builder's range Grout Colour: Grey grout colour - with mould inhibitor |
| WC Wall Tiles | Product: Ceramic Tile Brand: | Type: 450mm x 450mm Code: TBC Note: 1200mm height tiling to all walls | Tile Colour: Satin White Grout By: Builder's range Grout Colour: Black to floors & walls |
| Deck (Master Suite) | Product: Ceramic Tile Brand: | Type: 600mm x 600mm floor & skirting Code: TBC Note: Half height skirting tile where floor abuts cladding material | Tile Colour: Grey Grout By: Builder's range Grout Colour: Grey |
| Deck (Bed 3) | Product: Ceramic Tile Brand: | Size: 600mm x 600mm floor & skirting Note: Half height skirting tile where floor abuts cladding material | Tile Colour: Grey Grout By: Builder's range Grout Colour: Grey |
| Alfresco | Product: Ceramic Tile Brand: | Size: 600mm x 600mm floor & skirting Note: Half height skirting tile where floor abuts cladding material | Tile Colour: Grey Grout By: Builder's range Grout Colour: Grey |
| CAULKING - SILICON |  |  |  |
| Generally | Product: Antifungal high silicon content Brand: Commercial | Locations: All floor wall joints, bench to fixture joints. Reference: Refer to Specification S18 Tiles & Paving & S18A Silicon & Caulking for details | Colour: as close to tile colour as possible (depends on caulk colour availability through manufacturer) |
| FLOORING |  |  |  |
| Main Floor Areas | Product: Ceramic Tile Brand: TBC | Size: 600mm x 600mm floor Note: Half height skirting tile where floor abuts cladding material | Tile Colour: TBC Grout By: Builder's range Grout Colour: Grey |
| Main Floor Areas - floor to wall joint | Material: Caulked | Type: Mold Resistant Silicon | Colour: Clear |
| Carpet | Manufacturer: Builder's range Material: Medium Specification Range | Location: Media, Stairs, Area around stairs, Master Suite, WIR, BED 2, Hallway, BED 3, Bed 4 Type: Patterned Underlay: Premium Quality | Budget $45/m2 plus GST Colour: TBC (Dark preferred) |
| PAINT SELECTION |  |  |  |
| GENERALLY |  |  |  |
| Interior Paint System | Brand: Builder's range | Location: Walls - Three (3) coat internal paint system, tinted sealer and two top coats using professional-grade paints Location: Ceilings - Three (3) coats to ceiling using a professional-grade ceiling paint system, tinted sealer and two top coats | Finish: To paint manufacturer's standard specifications Colours: 2 colours allowed (ie: 1 colour to walls & 1 colour to timber work) Note: light, dark or accent (bright) colours require additional coats & base colour changes over and above a three coat system - additional costs apply if selected. |
| Exterior Paint System | Brand: Builder's range | Location: Eaves, Gables, Trims - Two (2) coat exterior paint system to external trim and doors using professional paints | Finish: To paint manufacturer's standard specifications Colours: 1 colour allowed except to gable trims or where specified, described and priced accordingly. Note: light, dark or accent (bright) colours require additional coats & base colour changes over and above a three coat system - additional costs apply if selected. |
| EXTERNAL PAINTING |  |  |  |
| Eaves Linings | Brand: Builder's range Product: | Colour: Lexicon | Finish: Washable Premium Acrylic Matt Note: Finish with 10mm nap roller |
| Lead Flashing | Brand: Builder's range Product: | Colour: Colorbond Surfmist (as close to this colour) | Finish: Washable Premium Acrylic Matt Note: Finish with brush |
| Vent Pipe (if accessible) | Brand: Builder's range Product: | Note: Vent pipe can only be painted if it is safe to access at the time painters are onsite. No additional safety equipment allowances have been made to paint the vent pipe | Colour: If accessible, to match roof or cladding colour Finish: Gloss Acrylic Note: Finish with brush |
| Meterbox | Brand: Builder's range Product: | Colour: Lexicon | Finish: Washable Premium Acrylic Matt Note: Finish with 6mm nap roller |
| Telecommunications & conduits in general | Brand: Builder's range Product: | Colour: Lexicon | Colour: Lexicon Finish: Washable Premium Acrylic Matt Note: Finish with brush |
| Front Entry Door & Sidelight | Not Included |  |  |
| Front Entry Door Frame | Not Included |  |  |
| Garage Access Door & Sidelight | Not Included |  |  |
| Garage Access Door Frame | Not Included |  |  |
| Downpipes | Not Included | Note: Natural, zinc-aluminium coated finish |  |
| Downpipe Shoes | Brand: Builder's range Product: | Colour: Lexicon | Finish: Washable Premium Acrylic Matt Note: Finish with brush |
| Leaf Catchers | Not Included |  | Finish: Washable Premium Acrylic Low Sheen |
| Droppers above Leaf Catchers | Not Included |  | Finish: Washable Premium Acrylic Low Sheen |
| Rainwater Tank Pipework | Not Included |  | Finish: Washable Premium Acrylic Low Sheen |
| Spa Pump Cover | Not Included |  |  |
| Garage Door Frame Timbers | Not Included | Cladding wraps from external corner back into house where it terminate to an external trim, plasterboard abuts trim | Finish: Washable Premium Acrylic Matt Note: Finish with 6mm nap roller |
| Alfresco Ceiling & vertical sheeting (adjoins eave) | Brand: Builder's range Product: | Colour: Lexicon | Finish: Washable Premium Acrylic Flat Note: Finish with 10mm nap roller |
| Deck Ceiling & vertical sheeting (adjoins eave) | Brand: Builder's range Product: | Colour: Lexicon | Finish: Washable Premium Acrylic Flat Note: Finish with 10mm nap roller |
| External Wall Cladding & Trims | Brand: Builder's range Product: | Colour: Lexicon | Finish: Washable Premium Acrylic Matt Note: Finish with 10mm nap roller |
| Gatehouse Ceiling & Trims | Brand: Builder's range Product: | Colour: Lexicon | Finish: Washable Premium Acrylic Flat Note: Ceiling, finish with 10mm nap roller. Trims, finish with brush |
| Masonry fences | Brand: Builder's range Product: | Colour: Lexicon | Finish: Washable Premium Acrylic Matt Note: Finish with 14mm nap roller |
| External stair handrail (TH1) | Product:External quality clear Substrate: Stain grade timber - Merbau | Colour: Clear Coats: 3 coats | Finish: Clear, low sheen Note: Finish with paint brush |
| External stair stringers | Products: Galvanised steel stringer (pair) | Colour: Raw, unpainted | Finish: No painting required |
| External stair landing face | Products: Jarrah | Colour: Clear | Finish: Clear, low sheen Note: Finish with paint brush |
| External stair treads | Products: Jarrah | Colour: Clear | Finish: Clear, low sheen Note: Finish with paint brush |
| INTERNAL PAINTING |  |  |  |
| NOTE: Oil based primer/undercoat to all pine products to reduce the risk of swelling of timber grains sometimes associated with water based primer or undercoats |  |  |  |
| Ceilings | Brand: Builder's range Product: | Colour: Lexicon Quarter Locations: All interior ceilings, Alfresco, Porch, Gatehouse, Deck, Bed 3 Deck | Finish: Washable Premium Acrylic Flat Note: Finish with 10mm nap roller |
| Bulkheads | Brand: Builder's range Product: | Colour: Lexicon Quarter Locations: Entry, Media, Kitchen, Bed 2, Bed 3, Bed 4, Master Suite | Finish: Washable Premium Acrylic Flat Note: Finish with 10mm nap roller |
| Cornice | Brand: Builder's range Product: | Colour: Lexicon Quarter | Finish: Washable Premium Acrylic Flat Note: Finish with brush where required |
| Walls | Brand: Builder's range Product: | Colour: Lexicon Quarter | Finish: Washable Premium Acrylic Low Sheen Note: Finish with 10mm nap roller |
| Door frames | Brand: Builder's range Product: | Colour: Lexicon Quarter | Finish: Washable Premium Acrylic Gloss Note: Finish with paint brush |
| Window frames | Brand: Builder's range Product: | Colour: Lexicon Quarter | Finish: Washable Premium Acrylic Gloss Note: Finish with paint brush |
| Architraves & Door headers (including sides/edges) | Brand: Builder's range Product: | Colour: Lexicon Quarter | Finish: Washable Premium Acrylic Gloss Note: Finish with paint brush |
| Skirtings | Brand: Builder's range Product: | Colour: Lexicon Quarter | Finish: Washable Premium Acrylic Gloss, Note: Finish with paint brush |
| Internal doors | Brand: Builder's range Product: | Colour: Lexicon Quarter | Finish: Washable Premium Acrylic Gloss Note: Finish with 4mm nap roller |
| Tops, bottoms & sides of internal doors | Brand: Builder's range Product: | Colour: Lexicon Quarter | Finish: Washable Premium Acrylic Gloss Note: Finish with 4mm nap roller Note: Primer and two top coats, three coats to all edges |
| Internal stair balustrade newel posts | Brand: Builder's range Product: NOT INCLUDED | Colour: Lexicon Quarter Location: Stairs | Finish: Washable Premium Acrylic Gloss Note: Finish with paint brush |
| Internal stair handrail (HR1) | Product: Internal quality clear Substrate: Stain grade timber - Jarrah | Colour: Clear | Finish: Clear, low sheen Note: Finish with paint brush |
| Internal stair stringer | Brand: Powdercoat Product: | Colour: Black | Finish: Powdercoated |
| Internal stair landing | Product: Internal quality clear Substrate: Stain grade timber - Jarrah | Colour: Clear | Finish: Clear, low sheen Note: Finish with paint brush |